STUDY ON THE GROUNDWATER POTENTIAL EVALUATION AND MANAGEMENT

PLAN FOR THE SOUTHEAST KALAHARI (STAMPRIET) ARTESIAN BASIN

IN THE REPUBLIC OF NAMIBIA

## Japan International Co-operation Agency

### JICA Study Team

# EVALUATION OF TEST PUMPING DATA

Borehole

J1-A (WW 39839)

Christiana

| Box 86386 Windhoek Namibia | METZGER PM DRILLING Box 11733 Windhoek Namibia |
| --- | --- |

Windhoek

June 2000

- PUMPING TEST ANALYSIS

J1-A (WW39839) - Pumping well

2318BD-35- Observation well

1.1.	Well Efficiency (Annex 1)

Well Efficiency was analysed by making use of the Jacob method for draw down data.  The well efficiencies at the range of pumping rates are summarised in Table 2 below.

Table 1: Borehole efficiency at various pumping rates

| Borehole number | Step | Abstraction Rate [m3/h] | Draw Down* [m] | Borehole Efficiency [%] |
| --- | --- | --- | --- | --- |
| J1-A | 1 | 3.7 | 2.69 | 53.4 |
|  | 2 | 6.23 | 8.26 | 48.3 |
|  | 3 | 9 | 9.85 | 43.7 |
|  | 4 | 12.1 | 15.4 | 39.5 |
|  | 5 | 15 | 19.8 | 36.3 |

* modelled draw down using the Jacob analysis method

Data on the linear and non-linear well losses and skin factors as well as the efficient well radius are presented in Annex 1.

1.2.	Constant Discharge Test Analysis (Annex 2 - 5)

The draw down curve indicates leaky conditions.  Hantush model for leaky aquifers was used to simulate and verify the actual data and analysis approach.  Simulation parameters used in simulation were obtained from the analysis of the pumping test graphs.  Storativity was estimated due to the fact that none of the 2 observation boreholes are open or located in the tested aquifer.

Table 2: Aquifer Parameters calculated for J1-A; Auob sandstone

| Borehole number | Analysis Method | T | s | k | S | Simulation model | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [m2/day] | [m] | [cm/sec] | [-] |  |  |
| J1-A | Walton-Hantush -draw down | 25,2 | 75 | 3.9 x 10-4 | *1 x 10-5 | Hantush | Poor correlation between pumping rates and water level e.g. water level rises during abstraction. |
|  | Walton-Hantush -recovery | 25,3 | 75 | 3.9 x 10-4 | *1 x 10-5 |  |  |

For leaky aquifers, the Walton Hantush analysis method with draw down and recovery data was once again used to calculate the hydraulic conductivity of the aquifer and the aquitard (Annex 2 & 3).

The abstraction has no real influence on the observation borehole due to its distance from the abstraction borehole (see radius of influence R).  Because the observation borehole was pumped during the constant discharge test, the data is not reliable for evaluation.

The radius of influence (R) was estimated after SICHARDT (1928) using the equation:

R = 3000 x s x Kf 1/2

R = 3000 x 11.2 x 1.97 x 10-3 = 66 m

where

R = Radius of influence

s = Draw down in abstraction borehole at end of pumping

Kf = Permeability of the aquifer

The equation is approximately correct for unconfined aquifers. In case of a confined aquifer the radius of influence most probably larger and the 66 m are considered to be the minimum value.

A proper evaluation of R (and storativity S) will only be possible once reliable data from observation wells, penetrating the same aquifer as the pumped well, are available.

linear aquifer loss

linear well loss

non-linear well loss

J1-A

2318BD-35

Distance to observation borehole:  2318BD-35 = 400 m