STUDY ON THE GROUNDWATER POTENTIAL EVALUATION AND MANAGEMENT

PLAN FOR THE SOUTHEAST KALAHARI (STAMPRIET) ARTESIAN BASIN

IN THE REPUBLIC OF NAMIBIA

## Japan International Co-operation Agency

### JICA Study Team

# EVALUATION OF TEST PUMPING DATA

Boreholes

J4-K (WW39845)

Okonjama

| Box 86386 Windhoek Namibia | METZGER PM DRILLING Box 11733 Windhoek Namibia |
| --- | --- |

Windhoek

July 2000

1.	PUMPING TEST ANALYSIS

## J4-K (WW39845) - Pumping well

1.1.	Constant Discharge Test Analysis (Annex 1)

The borehole is screened in dolerite, baked shale and Kalkrand Basalt and not in the upper Kalahari sediments.  Transmissivity of the underlying Karoo Formation is very low, in the order of 0.5 m2/day and can be regarded as an aquitard.  The borehole is virtually dry and water is only pumped from storage in the aquifer. Only a slight recovery in the water level is observed.

The simulation results of the pumping tests are displayed in Annex 1.