STUDY ON THE GROUNDWATER POTENTIAL EVALUATION AND MANAGEMENT

PLAN FOR THE SOUTHEAST KALAHARI (STAMPRIET) ARTESIAN BASIN

IN THE REPUBLIC OF NAMIBIA

## Japan International Co-operation Agency

### JICA Study Team

# EVALUATION OF TEST PUMPING DATA

Borehole

J5-N (WW39848)

Maritzville - Stampriet

| Box 86386 Windhoek Namibia | METZGER PM DRILLING Box 11733 Windhoek Namibia |
| --- | --- |

Windhoek

September 2000

- PUMPING TEST ANALYSIS

J5-N (WW39850) - Pumping well

#### No observation borehole

1.1.	Well Efficiency (Step Draw Down Test) (Annex 1)

Well Efficiency was analysed by making use of the Jacob method for draw down data. Aquifer parameters used for the calculation of well efficiency were obtained from the evaluation results of the constant discharge test, which is discussed in Section 1.2 below.

The well efficiencies at the range of pumping rates used during the step draw down test are summarised in Table 1 below. Only the first three steps could be evaluated for the calculation of the borehole efficiency. Due to the low abstraction rate the well bore storage affects the draw down data during the first 60 minutes of each 120 minutes test. The cut-off time was chosen at 60 minutes.

Table 1:	J5-N; borehole efficiency at various pumping rates

| Borehole Number | Step | Abstraction Rate [m3/h] | Draw Down* [m] | Borehole Efficiency [%] |
| --- | --- | --- | --- | --- |
| J5-N | 1 | 0.6 | 9.3 | 33.5 |
|  | 2 | 0.9 | 21.7 | 23.2 |
|  | 3 | 1.2 | 35.5 | 17.8 |

* modelled draw down using the Jacob analysis method

Data on the linear and non-linear well losses and skin factors as well as the efficient well radius are presented in Annex 1.

1.2.	Constant Discharge Test Analysis (Annex 2 - 3)

An abstraction rate of 0.8 m3/h was applied for the constant discharge test. The constant discharge draw down curve of abstraction borehole J5-N indicates confined conditions. For confined aquifers, the Theis analysis method with draw down data was used to calculate the hydraulic conductivity of the aquifer (Annex 2). The recovery data could not be evaluated due to the artesian conditions.

The aquifer storativity was estimated due to the fact that no observation boreholes are available.

The results of the constant discharge analysis are summarised in Table 2 below.

Table 2:	Aquifer Parameters calculated for J5-N; Nossob sandstone

| Borehole number | Analysis Method | T | s | kf | S | Simulation model | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [m2/day] | [m] | [cm/sec] | [-] |  |  |
| ##### J5-N | Theis-draw down | 1.2 | 25 | 5.5 x 10-5 | *3 x 10-5 | Theis | *Storativity estimated - Observation boreholes not located in the tested aquifer |

The Theis model for confined aquifer conditions was used to simulate and verify the actual data and analysis approach of the constant discharge test. Simulation parameters summarised in Table 2 were used in simulation of the actual pumping test data (See Annex 3 for simulation results).

The radius of influence (R) was estimated after SICHARDT (1928) using the equation:

R = 3000 x s x Kf 1/2

R = 3000 x 25.7 x 7.45 x 10-4 = 57 m

where

R = Radius of influence

s = Draw down in abstraction borehole at end of pumping

Kf = Permeability of the aquifer

The equation is approximately correct for unconfined aquifers. In case of a confined aquifer the radius of influence most probably larger and the 57 m are considered to be the minimum value.

A proper evaluation of R (and storativity S) will only be possible once reliable data from observation wells, penetrating the same aquifer as the pumped well, are available.

Linear aquifer loss

Linear well loss

Non-linear well loss

Linear aquifer loss

Linear well loss

Non-linear well loss

Data on the linear and non-linear well losses and skin factors as well as the efficient well radius are presented in Annex 2.1.

Distance of observation boreholes:   J6-K = 63.5 m;    J6-N = 58.5 m