STUDY ON THE GROUNDWATER POTENTIAL EVALUATION AND MANAGEMENT

PLAN FOR THE SOUTHEAST KALAHARI (STAMPRIET) ARTESIAN BASIN

IN THE REPUBLIC OF NAMIBIA

## Japan International Co-operation Agency

### JICA Study Team

# EVALUATION OF TEST PUMPING DATA

Boreholes

J7-N (WW 39853)

Jackalsdraai

| Box 86386 Windhoek Namibia | METZGER PM DRILLING Box 11733 Windhoek Namibia |
| --- | --- |

Windhoek

October 2000

1.	PUMPING TEST ANALYSIS

## J7-N (WW39853) - Pumping well

1.1.	Well Efficiency (Step draw down test) (Annex 1)

Well Efficiency was analysed by making use of the Jacob method for draw down data. Aquifer parameters used for the calculation of well efficiency were obtained from the evaluation results of the constant discharge test, which is discussed in Section 1.2 below.

The well efficiencies at the range of pumping rates used during the step draw down test are summarised in Table 1 below. Only four of the two steps were utilised for the evaluation of the borehole efficiency. The third step was interrupted due to pumping problems caused by accumulation of gas in the pump. The well bore effect is dominant throughout the test and only the last 10 minutes of each 120 minutes step was used for the evaluation.

Table 1:  J7-N; Borehole efficiency at various pumping rates

| Borehole number | Step | Abstraction Rate [m3/h] | Draw Down* [m] | Borehole Efficiency [%] |
| --- | --- | --- | --- | --- |
| J7-N | 1 | 0.3 | 18.1 | 86.6 |
|  | 2 | 0.6 | 45.2 | 69.3 |

* modelled draw down using the Jacob analysis method

Data on the linear and non-linear well losses and skin factors as well as the efficient well radius are presented in Annex 1.

1.2.	Constant Discharge Test Analysis (Annex 2 - 3)

The constant discharge draw down curve of abstraction borehole J7-N indicates leakage. For leaky aquifers, the Walton Hantush analysis method with draw down and recovery data was once again used to calculate the hydraulic conductivity of the aquifer and the aquitard (Annex 2 & 3).

Aquifer storativity was estimated due to the fact that no suitable observation boreholes was available.  The constant discharge test was implemented at a rate of 0.3 m3/h and was stopped after 9 hours, due to a draw down of 66 m. The water level was close to pump inlet depth.  The  water level recovered fully after three days. The recovery curve was evaluated using the Theis recovery method.

The results of the constant discharge analysis are summarised in Table 2 below. The transmissivity of the test pumping is compared to the results of the slug test. Again, the transmissivity of the slug test is slightly higher, due to the bigger influence of the gravel pack and the disrupted formation surrounding the borehole. The evaluated very low values of T = 0.01 and 0.06 m2/day are, however, in the same order of magnitude.

Table 2:  Aquifer Parameters calculated for J7-N; Nossob sandstone

| Borehole number | Analysis Method | T | s | k | S | Simulation model |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | [m2/day] | [m] | [cm/sec] | [-] |  |
| J7-N | Theis recovery | 0.01 | 17 | 6.8 x 10-7 | *8 x 10-4 | Theis |
|  | Slug test (for comparison) | 0.06 | 17 | 2.1 x 10-6 | N/A | Cooper et al |

*Estimated

The Theis model for confined aquifer conditions was used to simulate and verify the actual data and analysis approach of the constant discharge test.  Simulation parameters summarised in Table 2 were used in simulation of the actual pumping test data (See Annex 3 for simulation results).

The radius of influence (R) was estimated after SICHARDT (1928) using the equation:

R = 3000 x s x Kf 1/2

R = 3000 x 66.61 x 8.3 x 10-5 = 17 m

where

R = Radius of influence

s = Draw down in abstraction borehole at end of pumping

Kf = Permeability of the aquifer

The equation is approximately correct for unconfined aquifers. In case of a confined aquifer the radius of influence most probably larger and the 17 m are considered to be the minimum value.

A proper evaluation of R (and storativity S) will only be possible once reliable data from observation wells, penetrating the same aquifer as the pumped well, are available.

linear aquifer loss

linear well loss

non-linear well loss

J4-K

Linear aquifer loss

Linear well loss

Non-linear well loss